Cong thuc toi uu demo combat LCD STM32

Dung lai khi demo bi lag, nhan vat nhay lam meo background, hoac dirty-rect ve qua nang.

Y tuong chinh: build background cache 320x240 mot lan luc init, sau do moi dirty-rect copy pixel nen tu RAM thay vi tinh lai nen tung pixel moi frame.

# 1. Trieu chung

Demo combat bi lag khi co 2 nhan vat + effect.

Khi nhan vat nhay cao, vung nen phia tren bi meo/xoa sai, dac biet may, mat troi, nui.

Sau khi them Chidori/Getsuga, dirty-rect phai ve nhieu hon nen frame rate tut ro.

# 2. Nguyen nhan goc

Khong nen moi frame goi Battle_GetBackgroundPixel() voi nhieu nhanh if cho tung pixel trong dirty-rect.

Neu gop tat ca object vao mot dirty-rect lon, man hinh phai redraw ca khoang giua hai nhan vat, rat ton bus LCD.

Ham khoi phuc background phai khop 100% voi ham ve background ban dau. Neu ve nui theo block 8px luc init nhung tinh theo x tung pixel luc restore thi se bi bien dang.

# 3. Cach fix da ap dung

| Hang muc | Cach lam | Vi tri code |
| --- | --- | --- |
| Cache background | Them s_backgroundPixels[LCD_PORT_WIDTH * LCD_PORT_HEIGHT], build mot lan bang Battle_BuildBackgroundCache(). | Core/Src/battle_demo.c |
| Redraw nhanh | Trong Battle_DrawDirtyRect(), copy nen tu s_backgroundPixels vao s_compositeRun thay vi goi Battle_GetBackgroundPixel() moi pixel. | Battle_DrawDirtyRect |
| Dirty-rect nho | Dung Battle_DrawSnapshotChange() cho tung object rieng, khong gop player/cpu/effect thanh mot rect lon. | Battle_UpdateActors |
| Khoi phuc nen dung | Battle_GetBackgroundPixel() phai khop background goc: nui theo block 8px, cloud/sun/tile/wall dung mau. | Battle_GetBackgroundPixel |
| Frame timing | Quet frame theo durationMs cua moveset: rawFrame = elapsed / frameDuration; state finished = tong duration tung frame. | Core/Src/combat_actor.c |

# 4. Mau code can nho

static uint16_t s_backgroundPixels[LCD_PORT_WIDTH * LCD_PORT_HEIGHT];

static void Battle_BuildBackgroundCache(void)
{
  for (uint16_t y = 0U; y < LCD_PORT_HEIGHT; y++)
  {
    for (uint16_t x = 0U; x < LCD_PORT_WIDTH; x++)
    {
      s_backgroundPixels[((uint32_t)y * LCD_PORT_WIDTH) + x] =
          Battle_GetBackgroundPixel(x, y);
    }
  }
}

// Trong Battle_DrawDirtyRect:
s_compositeRun[col] = s_backgroundPixels[((uint32_t)y * LCD_PORT_WIDTH) + x];

# 5. Checklist khi sau nay lam lai

Giu BATTLE_TICK_MS khoang 33ms neu muon nh?p giong commit demo Naruto muot.

Khong full-screen redraw moi frame.

Khong tinh background tung pixel trong realtime loop neu background phuc tap.

Neu them effect/projectile moi, dua no vao snapshot/dirty-rect chung thay vi ve-rieng/xoa-rieng.

Ham Battle_GetBackgroundPixel() phai reproduce duoc moi layer nen da ve trong Battle_DrawBackground().

Sau moi thay doi build bang make -C Debug all va test nhay cao qua vung may/nui.

# 6. Trade-off

Cache nen ton RAM: 320 * 240 * 2 = 153600 bytes. STM32F429ZITx co RAM 192KB nen van build duoc trong project nay. Neu sau nay bss qua cao, can can nhac chuyen cache sang CCMRAM hoac cache theo tile/scanline.

# 7. Ket qua hien tai

Build sau toi uu: text=924564, data=20, bss=157620.

Demo hien tai: P1 Naruto, dummy Sasuke.

Da fix nen bi meo khi nhay va giam lag bang background cache + dirty-rect nho.